| # CH | INPUT | # MICROPHONE | FOH INSERT | MON | STAND |
| --- | --- | --- | --- | --- | --- |
| 1 | Kick | Audix D6 | Gate 1 | Gate | Small |
| 2 | Guira | KM184 |  |  | Large |
| 3 | Bongo L | Sennheiser 421 | Comp 1 |  | Large tele |
| 4 | Bongo R | Sennheiser 421 | Comp 1 |  | Large tele |
| 5 | Bass | Active d.i (Countryman) | Comp 3 |  | guitar stand |
| 6 | Rhythm guitar 1 | Active d.i (Countryman) |  |  | guitar stand |
| 7 | Lead guitar 1 | Active d.i (Countryman) | Comp 4 |  | guitar stand |
| 8 | Lead Vocal 1 | Shure KSM9 | Valve comp+Gate |  | Large tele |
| 9 | Backing Vocal 1 | Sennheiser e935 | Comp 5 |  | Large tele |
| 10 | Backing Vocal 2 | Sennheiser e935 | Comp 5 |  | Large tele |
| 11 | Backing Vocal 3 | Sennheiser e935 | Comp 5 |  | Large tele |

| 1 x Audix D6 |  | 1 x small teleboom stands | Foh sound: Benjamin email ben@iasorecords.com Mobile + 1 917-693-1808 |
| --- | --- | --- | --- |
| 3 x Sennheiser e935 |  | 6 x large teleboom stands |  |
| 2 x Sennheiser 421 |  | 3 x guitar stands |  |
| 1 x Km184 |  | 1 x Shure KSM9 |  |
|  |  |  | 1 x Beyerdynamic M88 (spare in case ksm9 not good on lead v) |

General Mix Instruction:. High pass rhythm guitar at about 150Hz. High pass Guira at about 350Hz with another cut around 1-2kHz. Kick drum not too bassy so doesn't clash with the bass guitar.

General Monitor Instructions: Lead guitar must stand out in all monitors. All backing vocalists need to be able to hear all the other backing vocalists as well as the lead vocal clearly. No kick drum in monitors.